Table of Contents

1   Introduction

1.1       Purpose

1.2       Document Conventions

1.3       Project Scope

2   Device Characteristics

2.1       Barcode For process Completion

2.2       Barcode For process Failed/Reworked

2.3       Push button for count

2.4	  LED indicators

3   Work Flow

3.1	Main Process

3.2	Steps

3.3 	Identification Marker

4   Requirements

5   Data Represented

5.1 Dashboard

5.2 Station status

5.3 Track status

5.4 Track Shipment

5.5 Statistical Data

5.6 Compare Builds

6   Notifications

6.3 Display Notification

6.3 SMS Notification

6.3 Email Notification

7   Test Cases

1. Introduction

1.2 Purpose:

A streamlined Data tracker. A means to track products, create different statistical data and real time information of different items manufactured.

Document Convention:

| Abbreviation | Word |
| --- | --- |

1.3 Project Scope:

Data tracking for a streamlined manufacturing process so that corrections can be made to make the production process more efficient.

2. Device Characteristics

2.1 Barcode for Process Completion:

Barcode Scanner is used to track the movement of a product in a production line. This barcode scanner is blue/green to indicate that it should be used only if a process is passed.

2.2 Barcode for Process Fail/Rejected:

Barcode Scanner 2 is used to track failures for each station. This helps to rectify recurring errors in a station. This barcode scanner is color coded in red to scan the barcode of a failed product.

2.3 Push Button for Count:

Push button can be used for processes that do not support barcode traceability feature.

2.4 LED indicators:

There are 3 lighting schemes associated with the device.

- Green-Indicates the barcode has been accepted by the device

- Red- The Barcode scanner not connected to the device

- Blue-device is in sync with the server

3. Work Flow

3.1 Main Process

- The production moves from Bare PCB through the production line to the Shipment with a barcode being attached on the bare PCB to identify each product

- After every stage, the product is scanned. For Burn in, product is scanned before and after the stage.

- If the product fails then the reject barcode scanner is used. The product is then sent to previous stages and brought back. Then scanned in the same order.

- Packing stage will have a PC to display and check if the product has gone through all the stages. Only after that it will be shipped and made as a passed product

- Shipping will have a pc and a form to submit the goods shipped

3.2 Steps

Stage 1: Bare PCB

- Barcode is added to the bare PCB

Device 1

Stage 2: SMT1

Device 2

After SMT1, The PCB is scanned to show that the particular PCB has completed SMT1.

Device 3

Stage 3: SMT2

After SMT2, The PCB is scanned to show that the particular PCB has completed SMT2.

Stage 4: Reflow

Device 4

After Reflow, The PCB is scanned to show that the particular PCB has completed Reflow.

Stage 5: Through-hole component mounting

Device 5

After each mounting, The PCB is scanned to show that the particular PCB has completed the stage.

Stage 6: Wave Soldering

Device 6

After each Wave Soldering, The PCB is scanned to show that the particular PCB has completed Soldering.

Stage 7 Inspection And Touch-up

Device 7

After each Inspection, The PCB is scanned to show that the particular PCB has been inspected.

Device 8

Stage 8: Cleaning

After each PCB is cleaned, The PCB is scanned to show that the particular PCB has completed cleaning.

Device 9

Stage 9: PCB Functional Test

After each PCB is Tested, The PCB is scanned to show that the particular PCB has been tested functionally.

Device 10

Stage 10: Wire Harness assembly

After each Wire Harness assembly, the product is scanned.

Device 11

Stage 11: Box assembly

After each PCB is boxed-in, The PCB is scanned to show that the particular PCB has been boxed-in.

Device 13

Device 12

Stage 12: Burn-in

Before and after The PCB is tested by burn-in, the product is scanned.

Stage 13: Sticker assembly

Device 14 121212

After Sticker is attached to the adapter, the product is scanned to show that sticker is attached

Stage 14: Packing

Device 15 121212

Device 16

After Sticker Assembly, product is checked in a pc if it has completed all the processes.

3.3 Identification Marker

  - Each PCB will have a 4 digit Precursor in their barcode to identify the product.

4. Requirements

- Devices:16

One device acts as pc in shipping while the rest are for the barcodes

- Server PC

A server pc is used to control all the devices and store relevant information

- Wi-Fi Router:1

To connect all the devices

- Repeaters- based on size of production

To boost the connection between all the devices in a large plant

- Barcodes:16(15 pass and 1 fail)

15 barcodes are used to track the normal    flow of manufacturing, while to track a faulty station 1 extra barcode can be attached to a device to track failures.

5. Data Represented

5.1 Dashboard

To get an overall idea of what is going on in the production setup

5.2 Station status

To get an on-a-glance view of the different stations, see if the devices are working, check failure rates for each device etc.

5.3 Track status

To track a particular product from its barcode. It shows all the stations a product passed through and at what time. This also displays if any product failed at a particular station

5.4 Track Shipment

To track a particular product from its barcode up to the packing stage so that the product can be packed and shipped

5.5 Statistical Data

Three main statistical data is displayed.

Failure per station- displays the failure rates for each station. In the above example, reflow failed 16 times. So we can correct the problems at that station.

Product Manufacture Efficiency- displays the efficiency of manufacturing each type of product

Failure per reason-to display what kind of faults are the most common.

5.6 Compare Builds

To compare between the different builds weekly, Daily or monthly and compare how much were completed/failed weekly daily or monthly.

6. Notifications

6.1 Display Notification

- Warning - particular device isn’t working

- Warning - failure rate of product is high

- Warning - particular station is faulty

- live data – each product finished

- live data – failure rate per station

6.2 SMS Notification

- Warning - device failure not solved

- Warning - failure of product not solved

- Warning - station fault not solved

6.3 Email Notification

- Warning - device failure rate high

- Warning - failure rate of product high

- Warning - station fault high

- EOD - Boards Completed

- EOD – Targets met

- EOD – failure rate

7. Test Cases

- Product is scanned more than once(rejected and accepted)- replaces the old data

- Even if product manufacturing order is different, Assurance of all process being completed till packing

- Stray barcodes are not saved(not having the same barcode precursor as a product being manufactured)

- If the product fails at one stage- the stage where it failed and reason for failure at that stage can be found

- Runs automatically when Wi-Fi, devices and PC is connected

- For the setup no constant internet connectivity is required. So no data lost when internet is off

- If any of the devices stop working for any reason, Client app displays it.

- Failure rate for specific product/station can be found

- Red LED glows when Barcode scanner is not connected or network problems

- Blue LED glows showing data is being transferred and green for barcode scanner picking up data

- Just first time setup

- USB port 1 is for normal barcode and port 2 for rejected barcode. They can be removed and inserted back at any time